PROJECT TITLE: MACHINE DOWNTIME REDUCTION IN AUTOMOBILE PARTS MANUFACTURING UNITS

## Company Background

## Auto Forge Components Pvt. Ltd. a Tier-2 automobile parts manufacturer in Coimbatore, produces steering brackets, oil pump housings, gearbox flanges, and engine mounting plates using CNC and VMC machines. Recently, the plant has faced increasing machine downtime affecting production targets.

## Problem Statement

## The company has failed to meet its daily production target of 2,000 units for the past three months due to high unplanned downtime, tool breakages, spindle overheating, and operator-related issues.

## Objectives

  - Identify machines responsible for high downtime

  - Understand planned vs. unplanned downtime

  - Perform shift and operator comparison

  - Analyze root causes using machine-level data

  - Recommend improvements

- Dataset Overview

A realistic dataset was constructed based on real-world manufacturing downtime patterns including planned/unplanned downtime, shift details, operator data, and production output.

- Analytical Methods

Excel-based analysis was performed including

- Data cleaning

- Formula calculations (downtime %, achievement %)

- Pivot tables for machine, shift, operator performance

- Dashboard charts for management review

- Scope of the study

- The data covers CNC machines for 2 days

- Includes productivity, downtime, and achievement status

- Does NOT include cost analysis or maintenance logs

- Data sets and Analysis

- Date

- Shift

- Operator

- Machine

- Total Produced

- Planned Down Time

- Unplanned Down Time

- Achievement %

| Date | Shift | Operator | Machine | Total Produced | Planned Down (mins) | Unplanned Down (mins) | Target | Major Issue | Achievement | Achievement (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 05-Jan | A | Rajesh | CNC-1 | 410 | 15 | 20 | 500 | Tool wear | Not achieved | 82.00% |
| 05-Jan | A | Manoj | CNC-2 | 430 | 10 | 25 | 500 | Alignment issue | Not achieved | 86.00% |
| 05-Jan | B | Siva | CNC-3 | 350 | 20 | 60 | 500 | Spindle overheating | Not achieved | 70.00% |
| 05-Jan | B | Karan | CNC-4 | 340 | 15 | 70 | 500 | Tool breakage | Not achieved | 68.00% |
| 05-Jan | C | John | VMC-1 | 300 | 30 | 40 | 500 | Coolant issue | Not achieved | 60.00% |
| 06-Jan | A | Rajesh | CNC-1 | 400 | 10 | 22 | 500 | Tool wear | Not achieved | 80.00% |
| 06-Jan | C | Manoj | CNC-2 | 420 | 15 | 20 | 500 | Alignment issue | Not achieved | 84.00% |
| 06-Jan | A | Siva | CNC-3 | 360 | 18 | 55 | 500 | Spindle overheating | Not achieved | 72.00% |
| 06-Jan | C | Karan | CNC-4 | 330 | 12 | 68 | 500 | Tool breakage | Not achieved | 66.00% |
| 06-Jan | B | John | VMC-1 | 310 | 25 | 45 | 500 | Coolant issue | Not achieved | 62.00% |

- CALCULATIONS (Sum, Average, If, Count If, etc...)

| 8 hrs(480 min)-Tt DT(595) | AVG Tt produced | Sum of planned DT | Sum of Unplanned DT | count of "not achieved" | count of "achieved" |
| --- | --- | --- | --- | --- | --- |
| 115 | 365.00 | 170.00 | 425.00 | 10.00 | 0.00 |
| 115 | 365.00 | 170.00 | 425.00 | 10.00 | 0.00 |
| 115 | 365.00 | 170.00 | 425.00 | 10.00 | 0.00 |
| 115 | 365.00 | 170.00 | 425.00 | 10.00 | 0.00 |
| 115 | 365.00 | 170.00 | 425.00 | 10.00 | 0.00 |
| 115 | 365.00 | 170.00 | 425.00 | 10.00 | 0.00 |
| 115 | 365.00 | 170.00 | 425.00 | 10.00 | 0.00 |
| 115 | 365.00 | 170.00 | 425.00 | 10.00 | 0.00 |
| 115 | 365.00 | 170.00 | 425.00 | 10.00 | 0.00 |
| 115 | 365.00 | 170.00 | 425.00 | 10.00 | 0.00 |

PIVOT TABLE AND CHARTS

- Pivot Table 1 — Production vs. Target (Performance Pivot)

| Row Labels | Sum of Achieved | Sum of Not achieved | Sum of Total Produced |
| --- | --- | --- | --- |
| CNC-1 | 0 | 20 | 810 |
| CNC-2 | 0 | 20 | 850 |
| CNC-3 | 0 | 20 | 710 |
| CNC-4 | 0 | 20 | 670 |
| VMC-1 | 0 | 20 | 610 |
| Grand Total | 0 | 100 | 3650 |

- Pivot Table 2 — Downtime Analysis Pivot (Planned vs. Unplanned)

| Row Labels | Sum of Planned Down (mins) | Sum of Unplanned Down (mins) |
| --- | --- | --- |
| CNC-1 | 25 | 42 |
| CNC-2 | 25 | 45 |
| CNC-3 | 38 | 115 |
| CNC-4 | 27 | 138 |
| VMC-1 | 55 | 85 |
| Grand Total | 170 | 425 |

FINDINGS

- All machines show 0 achieved output, indicating targets were not met.

- Each machine has 20 counts under “Not Achieved”, showing consistent underperformance across all units.

- CNC-3 and CNC-4 show higher actual downtime compared to their planned downtime.

- VMC-1 has the highest planned downtime (55) among all machines.

- Total production across machines is 3650 units, which is lower than the required output.

- The difference between planned DT and actual DT suggests poor maintenance planning or unexpected machine issues.

- Performance deviation is similar across machines, showing a system-level production issue, not just one machine.

- DASHBOARD

| Machine | Sum of achieved | Sum of not achieved | Sum of Total produced |  |  |  |  | Machine | sum of planned DT | Sum of un planned DT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CNC-1 | 0 | 20 | 810 |  |  |  |  | CNC-1 | 25 | 42 |
| CNC-2 | 0 | 20 | 850 |  |  |  |  | CNC-2 | 25 | 45 |
| CNC-3 | 0 | 20 | 710 |  |  |  |  | CNC-3 | 38 | 115 |
| CNC-4 | 0 | 20 | 670 |  |  |  |  | CNC-4 | 27 | 138 |
| VMC-1 | 0 | 20 | 610 |  |  |  |  | VMC-1 | 55 | 85 |
| Grand Total | 0 | 100 | 3650 |  |  |  |  | Grand Total | 170 | 425 |

- SUGGESTIONS

- Reduce Machine Downtime
Planned and unplanned downtimes must be minimized through preventive and predictive maintenance schedules.

- Set Realistic and Machine-Specific Production Targets
All machines currently show 0 target achievement, meaning targets may be set too high or not aligned with machine capability.

- Improve Monitoring and Reporting Systems
Real-time dashboards and daily performance reports should be used to track output, downtime, and productivity.

- Enhance Operator Skills and Maintenance Coordination
Regular operator training and better coordination between operators and maintenance teams can reduce errors and delays.

- CONCLUSION

The analysis shows that none of the machines achieved their production targets, mainly due to high downtime and operational inefficiencies. The consistent gap across all machines indicates systemic issues in planning, maintenance, and performance monitoring. Addressing these gaps through better maintenance practices, realistic target setting, and continuous monitoring will help improve machine productivity and overall output.